| ALPHA AEROSOLS Kot Abdul Malik | DOCUMENT: Job Order Card | DOC #: AER-JOC-001 DATE: July 08, 2026 REVISION: 00 |
| --- | --- | --- |
| Title: Production Job Order |  |  |

## 1. Job / Order Details

| Job Order #: | AO-2607-001 | Machine: | Aero Line 1 |
| --- | --- | --- | --- |
| Customer: | Aerosol Customer | Batch / Lot: | B-0708-A |
| Product Name: | Aerosol Can 45mm | Quantity: | 50,000 |
| Product ID: | 9001 | Diameter: | 45 mm |

## 2. Material Requirements (BOM)

*Calculations based on target Quantity to Produce. Sign-off confirms materials staged and verified.*

| P.ID | BOM ID | Item ID | Item Name | UOM | Net Qty/1000 | Waste | Total Gross Qty |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9001 | 100 | 501 | 45mm Aluminum Slug | kg | 20.000 | 10.0% | 1,111.11 |
| 9001 | 100 | 502 | SAPILUB LUBRIMET GR8 | kg | 0.021 | 10.0% | 1.17 |
| 9001 | 100 | 503 | SAPILUB ALULIQUID 13 | kg | 5.000 | 10.0% | 277.78 |
| 9001 | 100 | 504 | SCHEKOSOL INT PROT Gold (400 9 901) | kg | 1.045 | 10.0% | 58.06 |
| 9001 | 100 | 505 | SCHEKOSOL INT BEIGE (400 4 902) | kg | 1.140 | 10.0% | 63.33 |
| 9001 | 100 | 506 | SCHEKOSOL WH BC (422 0 903) | kg | 0.973 | 10.0% | 54.06 |
| 9001 | 100 | 507 | SCHEKOSOL CLEAR BC (422 9 918) | kg | 0.701 | 10.0% | 38.94 |
| 9001 | 100 | 508 | SCHEKOSOL OPV GLOSSY (422 9 900) | kg | 0.430 | 10.0% | 23.89 |
| 9001 | 100 | 509 | SCHEKOSOL OPV SILKMATT (422 9 903) | kg | 0.385 | 10.0% | 21.39 |
| 9001 | 100 | 510 | SunAltec MB PLUS 169766 OPAQUE WHITE | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11069 | UV INK YELLOW (168391) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11070 | UV INK MAGENTA (168396) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11071 | UV INK BLACK (401295) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11072 | UV INK TRANSPARENT WHITE (169167) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11065 | UV INK REFLEX BLUE (168402) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11066 | UV INK RHODAMINE (168397) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11067 | UV INK PURPLE (168398) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11068 | UV INK VIOLET (168399) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11074 | UV INK RED 032 (156358/CF01) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11073 | UV INK CYAN (168400) AEROSOL | kg | 0.280 | 10.0% | 15.56 |
| 9001 | 100 | 11075 | UV INK OPAQUE WHITE (400050) AEROSOL | kg | 0.280 | 10.0% | 15.56 |

## 3. Quality Inspection Results

| Visual / Appearance: |  |
| --- | --- |
| Dimensional Check: |  |
| Leak / Pressure Test: |  |
| QC Remarks: |  |

## 4. Department Sign-Off

| Planning | Production | Store | Quality |
| --- | --- | --- | --- |
| Signature & Date:___________________ | Signature & Date:___________________ | Signature & Date:___________________ | Signature & Date:___________________ |